# ECEN 215 – PRIN OF ELECTRICAL ENGR

# Spring 2019

## Lab 2:Node Voltages and Equivalent Circuits

Submitted by:

| Student Name | UIN: | Section # |
| --- | --- | --- |
| Daniel Lopez | 426001591 | 516 |
| Adam Johnston | 516007336 |  |
| Jared Blunt | 524009660 |  |

Date Performed: Feb 25th, 2019

- Objective

The objective of this lab was to find the node voltages of a given circuit and then use that information to find the values necessary to create a Thevenin equivalent circuit. We also compared these values to theoretical values and values obtained in a MultiSim simulation of the same circuit.

- Procedure

- Calculate the theoretical values of current and voltage for the initial given circuit, by hand

- On the breadboard, using resistors and wires, build the initial given circuit

- Using the multimeter, record the values of R1, R2, R3, R4, and RL

- Using the NI Analog Discovery as the voltage supply, record VS and IL with the multimeter

- Use these values to calculate and record the new values for the circuit

- Remove RL to measure and record ISC and V0V using the multimeter

- Simulate the initial circuit with the measured values to measure and record IL

- Difficulties

- Many of the resistors, boards, and other components were damaged or had different values than necessary.

- The wires were difficult to keep inside the Analog Discovery.

- Results

Initial Measurements

| Component | Measured Value |
| --- | --- |
| R1 | 47.5 Ohm |
| R2 | 46.1 Ohm |
| R3 | 98.0 Ohm |
| R4 | 217.2 Ohm |
| RL | 98.7 Ohm |
| Vsc | 4.996 V |

Circuit Measurements

| Component | Measured Value |
| --- | --- |
| IL | 10 mA |
| Vxy | 0.956 V |
| VL | 0.954 V |
| VL = IL x RL | 0.987 V |
| Error for VL (measured vs calculated) | 3.5% |
| VOC | 1.36 V |
| Isc | 39 mA |

Calculated Values

| Component | Calculated Value |
| --- | --- |
| Vth = Voc | 1.36 V |
| Rth = R2 // (R1 + (R3 // R4)) | 32.9 Ohm |
| Isc = Vth / Rth | 41.3 mA |
| IL = Vth / (Rth + RL) | 10.2 mA |

Simulated Comparison

| IL | Value |
| --- | --- |
| Simulated | 10.861 mA |
| Measured | 10.2 mA |
| Error | 6.48% |

Theoretical Values (from prelab)

| Component | Value |
| --- | --- |
| Rth | 33.4 Ohm |
| Vth | 1.44 V |
| IL (theoretical) | 10.8 mA |
| IL (measured) | 10.2 mA |
| Experimental Error | 5.6% |

Figure 1: Initial Circuit

Figure 2: Circuit with RL removed

Figure 3: Simulation of initial circuit to find IL  using measured values from the multimeter

Figure 4: Measured values and calculations using these measured values

- Conclusion

The experiment shows that calculations and simulations are beneficial in determining the ideal and approximate values for nodal analysis and in creating thevenin equivalent circuits. Physical testing of the circuit, though, allows for values that reflect all the variables found in the real world, such as the small resistance in wires and the breadboard. Because these methods can approximate and give the ideal values for circuits; they are crucial in their application for the real world, as they can give total acceptable loads and help to determine the direction and magnitudes of the currents in any given part of a circuit. However, in cases where extreme precision is needed, theoretical calculations alone will not suffice.